3.0  Results

3.2	Physical Habitat and Biological Assessment

3.2.1  	General Overview

Historic dredge mining operations have severely impacted the aquatic habitat of French Gulch resulting in the elimination of aquatic benthic communities in French Gulch downstream of the Wellington-Oro Mine site.  Compounding this problem are acutely toxic levels of D-Zn and D-Cd originating from the Mine.  Reduced trout numbers in the Blue River, below the confluence with French Gulch, is likely associated with both metal toxicity and poor aquatic-habitat conditions.  Segment 2 of the Blue River has been largely reclaimed in terms of installation of structures designed to increase stream flow and plantings for wetland and terrestrial improvements.  The ecosystem along this segment is substantially improved as compared to that found in French Gulch.  Nonetheless, stream flows in Segment 2 are and will continue to be a limiting factor to a healthy fishery in this segment, particularly in localized areas within the segment during seasonal low flows.

Figure 3.2-1a  Habitat in French Creek below FG-8

The paucity of aquatic biota in lower French Gulch is largely associated with the elimination of aquatic habitats (USEPA 2002) (Figures 3.2-1a and 3.2-1b).  Even with improved water quality resulting from the treatment of Wellington-Oro Mine discharges it is extremely unlikely that the aquatic habitat will ever be improved sufficient to support healthy trout populations (see Section 3.4 of this UAA).  However, improved habitat and water quality upstream of the Mine site support a population of Colorado River cutthroat trout.  CDOW is extremely concerned that water quality improvements in this segment will in result brown trout and other species migrating upstream to compete with the otherwise isolated population of native trout, in spite of several culverts that may act as physical barriers.  So, for reasons of economic feasibility and protection of the upstream native trout population it is not considered reasonable to attempt to develop a aquatic ecosystem along this approximately 1.4 mile segment of French Creek.  Further assessment of aquatic biologic systems is primarily focused on protecting the Blue River downstream from the confluence with French Creek.

Figure 3.2-1b  Habitat in French Creek below FG-5

3.2.2  	Effects of Dissolved Zinc (D-Zn) on Brown Trout Populations

Based on literature values (see Appendix B, Tables B3 and B4) for acute and chronic D-Zn toxicity levels in trout, it appears that brook trout have a higher tolerance of dissolved zinc (D-Zn) levels when compared to brown trout or cutthroat trout (Tables B3 and B4).  These data indicate that LC50’s for brook trout ranged from 1458 µg/L D-Zn (60 mg/L hardness) to 6980 µg/L (179 mg/L hardness).  Similarly, increased tolerance was also apparent in terms of a chronic response (Appendix B, Table B3).  Lethal values for cutthroat and rainbow trout ranged from 90-670 µg/L for cutthroat and from 93 (low hardness of 22 mg/L) to 4520 µg/L (high hardness of 350 mg/L) for rainbow trout (Table B3).   Studies conducted on the effects of D-Zn on brown trout indicated that these fish are more sensitive than brook trout; Marr (1994) reported that brown trout fry had a LC50 of 454 µg/L (50mg/L, hardness) and 1,000 µg/L D-Zn at 100 mg/L hardness (Appendix B, Table B4).

More recently, a number of experiments were conducted on the tolerance of brown trout to D-Zn (Davies and Brinkman 1999, Davies et al., 2000).  The results of these studies indicated that trout that were pre-exposed to D-Zn, as embryos, were more tolerant to D-Zn exposure than naïve brown trout (Table 3.2-1).  For example, trout pre-exposed to D-Zn had a chronic LC50 of 1329µg/L (50 mg/L hardness), in-contrast, un-acclimated trout had a chronic LC50 of 303 µg/L (Table 3.2-1).  However, regardless of pre-exposure, trout that were exposed once and allowed to depurate, and then exposed again showed significantly less tolerance to D-Zn (Table 3.2-1).  Chronic toxicity LC50’s were reduced from 1329 µg/L to 360 µg/L after a two-week period in clean water and further reduced to 342 µg/L after three weeks in clean water (Table 3.2-1).  It should also be noted that un-acclimated trout were also more sensitive to D-Zn after a two-week de-acclimation period; whereas initial chronic values of 303 µg/L were reduced to 194 µg/L D-Zn. Similar results were observed in terms of acute reactions (Table 3.2-1).

Table 3.2-1 -- Comparison of 96-hr LC50 D-Zn Concentrations from Acute and Chronic Toxicity Tests1

| Exposure | Acute 96-Hr LC50 (µg/L) | Chronic Value (µg/L) |
| --- | --- | --- |
| Un-acclimated | 871 | 303 |
| 200 Acclimated | 1397 | 1329 |
| 400 Acclimated | 1578 | 1329 |
| Two-week De-Acclimation PeriodA |  |  |
| Un-acclimated | 392 | 194 |
| 200 Acclimated | 561 | 360 |
| 400 Acclimated | 506 | 360 |
| Three-week De-Acclimation PeriodA |  |  |
| Un-acclimated | 392 | 194 |
| 200 Acclimated | 438 | 342 |
| 400 Acclimated | 384 | 342 |

1 With non-exposed (i.e., un-acclimated), and pre-exposed (i.e., acclimated to 200 µg/L and 400 µg/L of zinc) brown trout.  (A sub-chronic test periods were 18 and 13 days, respectively).[Note: Data were reproduced from Davies and Brinkman, (1999).]

These results are important, because acute and chronic LC50 values from standard tests, upon which biotic standards are typically developed, do not take this loss of acclimation into account and thus may not be as protective of aquatic life.  For example, resulting from increased dilution rates, during spring high-flows, D-Zn concentrations can decrease substantially  (see Section 3.3.5, Table 3.3-5).  Alternatively, increased tolerance to D-Zn may occur during low-flow and winter egg-incubation periods.  This increased tolerance can be rapidly lost during high flows, thus increasing brown-trout sensitivity to D-Zn.

Results from studies on early life stage (ELS) toxicity test indicated that the chronic value concentration of newly hatched brown trout fry was 381 µg/L D-Zn (Davies and Brinkman, 1999).  Marr (1994) reported an acute value of 454 µg/L D-Zn; both tests used water hardness at 50 mg/L (see Appendix B, Table B4).

#### 3.2.3 	D-Zn Criterion for Brown Trout

As outlined in Appendix B, Section B4.2, ambient water-quality criteria (AWQC) and associated table value standards (TVSs) are threshold concentrations that are designed to be protective of many different aquatic species under a variety of water-quality conditions (USEPA, 1986).  Table Value Standards (TVS) are water hardness dependent equations used to calculate acute and chronic toxicity thresholds (Appendix B, Table B1).  One objective of this UAA is to compile available literature on toxicity of metals to potential ecological receptors (primarily trout and invertebrates) for comparison with relevant State of Colorado table value standards (TVS) and water-quality conditions in French Gulch and Blue River sites. In an effort to achieve this objective, a D-Zn hardness based criteria for the effects on brown trout was used. This criteria are specific to a specific metal and the toxicity limits of species of interest; whereas, table value standards (Appendix B, Table B1) are set as a regulatory mechanism by the CDPHE’s Water Quality Control Commission (WQCC) to protect aquatic habitat (in terms of specific species) of a given stream.

In an attempt to define a site-specific standard for brown trout in this section of the Blue River, we used a recently developed D-Zn hardness based equation (Davies and Brinkman, 2003) as a criteria range. Based on the complete lack of suitable habitat in this section of the Blue River, in addition to, very low numbers of trout occurring at sites upstream from the confluence with French Creek, we contend that sensitive early life stages of trout are unlikely to occur in this section of the Blue River. Therefore, toxicity tests conducted with embryonically exposed or acclimated early life stages of trout (ELS) were selected as the best approach to represent site-specific conditions (Davies and Brinkman, pers. Comm).  The data used to develope this ELS equation are presented in Table 3.2-2.  In the event that aquatic habitat conditions may improve, or if upstream brown trout populations increase, thus increasing the opportunity for sensitive life stages of trout to occur in this stream section, a more restrictive equation non-ELS equation can be used as a site-specific standard (Davies and Brinkman, pers. Comm).

Based on regression analysis of these chronic data (Table 3.2-2) a significant positive result was obtained (p = 0.037) indicating a relationship between hardness levels and chronic toxicity of D-Zn.  The resultant equation from this regression analysis can be described as: Log10 (Brown Trout chronic) = Log10 (Intercept) + Regression Slope(Log10 Hard) (Davies and Brinkman, 2003).   Substituting the regression data on slope and line intercept from the analysis using acclimated fry into this format, a site-specific brown trout chronic criteria for D-Zn can be developed, and is expressed as the ELS Equation for D-Zn (Table 3.2-3).

## Table 3.2-2 – Data used to develop the Toxicity Criteria for Brown Trout in Colorado

| Toxicity Test | Hardness (mg/L) | Chronic Value (µg/L) |
| --- | --- | --- |
| ELS Fry | 26.8 | 162 |
| ELS Fry | 48.1 | 196 |
| ELS Fry | 54.1 | 381 |
| ELS Fry | 153 | 1306 |
| R2 = 0.927 | Constant/Slope = 0.347 | X Coeffient = 1.253 |

Sources: Davies and Brinkman (1999, 2002, and 2003).

## Table 3.2-3 – Dissolved zinc hardness based criteria for acclimatized early life stages of brown trout.

# D-Zn Hardness Based Equation for ELS

Log10 (Brown Trout chronic) = Log10 (0.347) + 1.253(Log10 Hardness).

Davies and Brinkman, 2003

##### Using this ELS equation we can then estimate chronic toxicity at various hardness levels; chronic toxicity and associated hardness levels are presented in Table 3.2-4.

Table 3.2-4.   Chronic brown trout criteria using the ELS equation for D-Zn

| Hardness (mg/L) | ELS Fry D-Zn Criteria (µg/L) | TVS (µg/L) |
| --- | --- | --- |
| 40 | 226 | 54 |
| 50 | 299 | 66 |
| 60 | 376 | 77 |
| 70 | 456 | 87 |
| 80 | 539 | 98 |
| 90 | 625 | 108 |
| 100 | 713 | 118 |

Data from: Davies and Brinkman, 2003

As outlined, an objective of this UAA is to compile available literature on toxicity of metals to and compare these findings with relevant State of Colorado table value standards (TVSs). The TVSs are threshold concentrations that are designed to be protective of many different aquatic species under a variety of water quality conditions.  Because TVSs are water hardness dependent equations, the average hardness conditions in the Blue River were used to calculate acute and chronic toxicity thresholds (Table 3.2-5).  Afterwards, by adjusting for changes in streamflows and hardness levels at the combined Blue River sites, acute AWQC values ranged between 87 and 109 µg/L D-Zn and chronic values ranged between 88 and 110 µg/L D-Zn (Table 3.2-5)

Table 3.2-5 -- Comparison of D-Zn Hardness Equation Criteria (ELS) with Table Value Standards for the Blue River Segment 2.

| ###### Criteria | ###### Zinc Concentrations |
| --- | --- |
| D-Zn Hardness Criteria @ 50mg/L | 299 µg/L |
| TVS Average Hardness Adjusted to 50mg/L | 65 µg/L (acute); 66 µg/L (chronic) |
| TVS Average Hardness High Flows | 87.0 µg/L (Ac.); 87.7 µg/L (chronic) |
| TVS Average Hardness Low Flows | 108.9 µg/L (Ac.); 109.8 µg/L (chronic) |

In conclusion, it is recommended that the aquatic-life protective level for D-Zn should be based on the ELS hardness criteria (Davies and Brinkman, 2003) outlined in Table 3.2-3. This approach assumes that physical habitat in these stream sections will restrict the trout population in terms of spawning and recruitment, and the lack of an upstream population will further compound these conditions. Therefore, it is probable, that sensitive trout fry will not occur in this stream section. We conclude that use of the more tolerant D-Zn hardness based equation (ELS) will be the pertinent standard for Segment 2 of the Blue River.

## 3.2.4   	Effects of Dissolved Zinc (D-Zn) on Macroinvertebrates

Studies conducted by Clements (1999) have revealed similar community patterns that may be useful in identifying key taxa and their toxicity levels.  First, mayflies, particularly Heptageniidae, are the most sensitive to heavy metals and typically the first taxonomic group eliminated at contaminated sites. Thus heptageniids such as Rithrogena sp., Epeorus sp., and Cinygmula sp. may serve as indicators of metal pollution at a site.  Early instars of these mayflies, as well as other taxa such as Drunella sp., have also shown to be particularly sensitive to metals and metal mixtures (Kiffney, 1995).  However, due to the paucity of acute/chronic LC50’s and/or no observed effect studies on macroinvertebrates, in addition to, extremely wide ranging results being reported in the literature (Table B9), developing protective criteria for macroinvertebrates is difficult.

Because of these limitations, we have used a community-level matrix in an attempt to provide estimated D-Zn concentration that will be protective of a macroinvertebrate community; these data are presented in (Table B10).  In effect, we use the abundance and/or presence or absence of Rhithrogena hageni, Epeorus longimanus, E. deceptivus, Cinygmula sp.(“Indicator mayflies”), and Baetis tricaudatus as the most reliable indicators for potential recovery at metal contaminated sites (Figure 3.2-2).  This fundamental principle implies that if theses most sensitive species are protected then other macroinvertebrate species will survive.  Based on these data we show that at a zinc concentration of less than 250 µg/L (Figure 3.2-2), sensitive and baetid mayflies become more abundant. Hardness values during these studies ranged from 43 to 91 mg/L.  Normalizing these data for hardness, the D-Zn 250 µg/L value would be reduced slightly, to 244 µg/L.  In reality, in order to restore a complete healthy diverse benthic community, D-Zn levels would have to be reduced to be less than 100 mg/L (Figure 3.2-2).  However, given some of the physical habitat constraints within these stream sections, it is unlikely, that a complete and diverse benthic community could exist, even in the absence of metal effects. Based on the results of our ELS D-Zn hardness based criteria for brown trout (Table 3.2-3), it is possible that some indicator species and other mayflies can survive at these levels (Figure 3.2-2).  We can then conclude that if the ELS D-Zn equation (Table 3.2-3) provides a limited protection to the brown trout population, then most macroinvertebrates will be protected using this ELS equation.

Figure 3.2-2 -- Relationship between D-Zn Concentrations and Mayfly Abundance in Different Colorado Sites [Note: Site hardness values ranged from 43 to 91 mg/L; see Appendix Table B10.]

### 3.2-5	Effects of Dissolved Cadmium (D-Cd) on Brown Trout Populations

Brown trout D-Cd toxicity values ranged between 0.4 and 6.7 µg/L D-Cd for chronic tests and varied from 1.4 and 2.16 µg/L D-Cd for acute tests (Table 3.2-6).  In an attempt to define criteria for brown trout that would be protective of adult brown trout, previously exposed to D-Cd, the literature values for the chronic effects that trout can endure were used (Table 3.2-7).  In a similar approach to D-Zn, a regression analysis to determine the relationship between hardness and chronic values for trout was used. The resultant equation is shown in Table 3.2-8.  The pooled slope for these trout data was 1.03, (r2 = 0.69).

Table 3.2-6 -- Reported literature values for brown trout D-Cd toxicity

[Note: NR = Data/information are not presented or known.]

| TAXA | AGE | HARDNESS | TIME | TYPE | LC50 (µg/L) | REFERENCE |
| --- | --- | --- | --- | --- | --- | --- |
| Brown trout | NR | 60 | ###### NR | acute | 2.16 | ERA, USEPA (2002) |
| Brown trout | NR | 50 | ###### NR | chronic | 5.004 | USEPA (2001) |
| Brown trout | NR | 39-48 | 96-hr | acute | 1.4 | Spehar and Carlson (1984) |
| Brown trout (wild) | Un-acclimated fingerlings | 39 | 96-hr | acute | 2.39 | Davies and Brinkman (1994b) |
| Brown trout (wild) | Un-acclimated fingerlings | 39 | 12 weeks | chronic | 0.4 | Davies and Brinkman (1994b) |
| Brown trout (hatchery) | Un-exposed as eggs; fingerlings | 40 | 96-hr | acute | 1.87 | Davies and Brinkman (1994b) |
| Brown trout (hatchery) | Pre-exposed as eggs; fingerlings | 40 | 96-hr | acute | 1.78 | Davies and Brinkman (1994b) |
| Brown trout (hatchery) | Un-exposed as eggs; fingerlings | 40 | 136 d | chronic | 1.3 | Davies and Brinkman (1994b) |
| Brown trout (hatchery) | Pre-exposed as eggs; fingerlings | 40 | 136 d | chronic | 1.3 | Davies and Brinkman (1994b) |
| Brown trout | Eyed eggs | 45 | ###### NR | chronic | 2 | Eaton et al. (1978) |
| Brown trout | Green eggs | 45 | ###### NR | chronic | 6.7 | Eaton et al. (1978) |

Table 3.2-7.  Trout data used to develop a hardness-based equation for chronic effects of D-Cd on acclimated adult brown trout.

| Study | Species | Life-stage | Hardness Mg/L | Chronic Value µg/L |
| --- | --- | --- | --- | --- |
| Brown et al. 1994 | Rainbow | 270d old | 250 | 4.31 |
| Sauter et al. 1976 | Brook | ELS | 37 | 1.732 |
| Eaton et al. 1978 | Brook | ELS | 44 | 2.04 |
| Brown et al. 1994 | Brown | Non-ELS | 250 | 16.49 |
| Davies and Brinkman, 1994 | Brown | Non-ELS | 40 | 1.3 |
| Davies and Brinkman, 1994 | Brown | Fry | 39 | 0.37 |
| Eaton et al. 1978 | Brown | ELS | 45 | 2 |
| ERA, 2002 | Brown | N/A | 60 | 2.16 |
| Chapman, 1978 | Rainbow | Swim-up | 23 | 1 |
| Roch and Maly, 1979 | Rainbow | N/A | 125 | 17.3 |
| Arillo et al. | Rainbow | N/A | 320 | 10 |
| Majewske and Giles, 1984 | Rainbow | N/A | 82 | 4.8 |
| Davies and Brinkman, 1987 | Rainbow | Fry | 29 | 1.26 |
| Davies and Brinkman, 1987 | Rainbow | Fry | 301 | 9.15 |
| Kumada et al. 1973 | Rainbow | N/A | 104 | 7 |

## Table 3.2-8.  Dissolved cadmium hardness based criteria for acclimated adult trout. Regression based on data from Table 3.2-7.

# D-Cd Chronic Hardness Based Equation for Acclimated Adult Brown Trout

Log10 (Trout chronic) = Log10 (1.031) – 1.45(Log10 Hardness).

##### Using the equation for D-Cd (Table 3.2-8) we can then estimate chronic toxicity at various hardness levels, chronic toxicity and associated hardness levels are presented in Table 3.2-9.

Table 3.2-9.   Chronic brown trout criteria using the equation for D-Cd.

| Hardness (mg/L) | Acclimated Adult Brown Criteria (µg/L) |
| --- | --- |
| 40 | 1.6 |
| 50 | 2.0 |
| 60 | 2.4 |
| 70 | 2.8 |
| 80 | 3.2 |
| 90 | 3.7 |
| 100 | 4.1 |

It should be the toxicity of D-Cd to aquatic life is dependent upon its relationship to the other minerals and metals in solution and therefore, these criteria may change.  In addition, Davies et al. (2000) suggested a D-Cd level of 1.2 ug/L D-Cd for the Animas River (hardness values for the Animas River in spring range between 42-82 mg/L).

These criteria (Table 3.2-9) are slightly higher than the D-Cd levels obtained from the TVS equations (Table 3.2-10 and Appendix B, Table B1).  Based on the TVS, acute and chronic values for trout would be 1.75 µg/L D-Cd (acute for trout) and 1.34µg/L D-Cd (chronic) both at a hardness of 50 mg/L (Table 3.2-10).  Our suggested standards (Table 3.2-9) exceed acutely lethal levels for brown trout, but accounting for poor habitat conditions and lack of potential for sensitive fry to occur, we suggest that these standards would be protective of acclimated adult brown trout.

Based on updated data and equations used for cadmium toxicity (USEPA 2001); current TVS standards may be become more restrictive in future years.  The revised cadmium equation and associated acute and chronic values are shown in Table 3.2-10.  These data (EPA 2001) indicate that brown trout are the most sensitive species for cadmium having an acute level of 1.6 µg/L for D-Cd (USEPA 2001).

Table 3.2-10.   Current TVS for D-Cd compared to future criteria for D-Cd.

| Hardness Mg/L | 1999 Chronic µg/L | 1999 Acute µg/L | 2001 Chronic µg/L | 2001 Acute µg/L |
| --- | --- | --- | --- | --- |
| 20 | 0.68 | 0.65 | 0.08 | 0.40 |
| 30 | 0.92 | 1.00 | 0.10 | 0.61 |
| 40 | 1.14 | 1.37 | 0.13 | 0.82 |
| 50 | 1.34 | 1.75 | 0.15 | 1.03 |
| 60 | 1.53 | 2.13 | 0.17 | 1.23 |
| 70 | 1.72 | 2.52 | 0.19 | 1.44 |

### 3.2.6	Effects of Dissolved Cadmium (D-Cd) on Macroinvertebrates

Invertebrate taxa appeared to be more tolerant to D-Cd than fish taxa (Table B16), with the exception of Daphnia, snails, amphipods, and small heptageniid mayflies (lowest estimated LC50 of less than 1.1 µg/L).  Harrahy (2000) estimated that 50% of Baetis tricaudatus mayflies died when exposed to between 2.2 and 11 µg/L D-Cd over several days.  Acute values in the same study were higher (between 29 and 41 µg/L D-Cd), and were similar to values reported in 96-hr tests by Williams et al. (1985) for Baetis rhodani..  For tests where hardness ranged from 40 to 70 mg/L, toxicity values ranged from less than 3 µg/L(chronic) to 2,278 µg/L (acute) (Table B16).  Toxicity values for stoneflies and caddis flies appeared to be high and similar to some Baetis values, but cannot be interpreted due to lack of data on hardness in toxicity tests (stonefly range: 26000 – 1300000 µg/L; caddis fly range: 200,000 – 1,100,000 µg/L).  It is concluded that, if D-Cd levels were maintained at the TVS levels for brown trout, then these levels would also be protective of a macroinvertebrate community.

#### 3.2.7	Effects of Lead (D-Pb) Toxicity to Brown Trout

There is a paucity of data for D-Pb toxicity to brown trout (Table 3.2-11) and other trout species (Appendix B, Table B17).  A “no-effect” concentration (NOEC) and lowest effect concentration (LOEC) based on mortality that occurred for brown trout between 60 and 105 µg/L D-Pb; however, deformities were observed as low as 45 µg/L D-Pb (Davies and Brinkman 1994c).   In terms of state TVS, the D-Pb values, after normalizing for site hardness are 36  µg/L (acute) and 1.4  µg/L (chronic) (WQCC, 2001). Based overall lack of data on D-Pb effects on brown trout (Table 3.2-11) the TVS concentrations to protect brown trout from chronic effects are recommended for purposes of this UAA.

## Table 3.2-11.  Reported literature values for D-Pb toxicity in brown trout

| TAXA | AGE | HARDNESS (mg/L) | TIME (months) | TYPE | LC50 (µg/L) |
| --- | --- | --- | --- | --- | --- |
| Brown trout | Fingerlings | 37 (alk=40) | 6-7 | Chronic- deformity | 45 |
| Brown trout | Fingerlings | 37(alk= 40) | 6-7 | Chronic-mortality | 79 |
| Brown trout | Fingerlings | 37(alk=40) | 6-7 | NOEC-LOEC | 60-105 |

#### (Data from Davies and Brinkman 1994c)

#### 3.2.8	Effects of Lead (D-Pb) Toxicity to Macroinvertebrates

Also similar to results for D-Cd and D-Zn, invertebrates appeared to be less sensitive to D-Pb than fish taxa (Appendix B, Table B18). Drunella mayfly chronic LC50’s were consistent across two studies (3500 µg/L D-Pb). The large-bodied stonefly Pteronarcys was less sensitive with chronic LC50’s ranging from 19200 – 20000 µg/L D-Pb.  Acute and chronic values reported for Tanytarsus chironomids ranged broadly (258µg/L chronic to 224000 µg/L D-Pb acute).  Harrahy (2000) reported that Chironomus midge were sensitive to lower levels of D-Pb in sediment (37.1 µg/L D-Pb)(Appendix B, Table B18).

3.2.9	Summary and Recommendations

Results from acute and chronic studies on D-Zn, D-Cd, and D-Pb varied widely for each taxa, regardless of similarities in water hardness and other study parameters.  Occasionally, key elements of the study design (length of time, organism age or stage, and water hardness) were not reported.  Furthermore, with the exception of D-Zn, the diversity of studies on toxicity of D-Pb and D-Cd were lacking. In general many toxicity tests are performed using daphnia and chironomid larvae and rainbow trout.  This variability and lack of information made it difficult to draw solid conclusions about how taxonomic groups differed in their responses to these metals.  General patterns included the fact that fish tented to be more sensitive to these metals than invertebrates, especially small fish or fish at early life-stages.

Based on the findings of this UAA the overall recommendations include adoption of a site-specific ELS hardness based criterion for D-Zn, similarly, a hardness based equation for acclimated brown trout as the chronic standard for D-Cd.   Finally, in terms D-Pb, the hardness-based equations outlined in the TVS be maintained as appropriate levels of protection (Table 3.2-12) are recommended.

Table 3.2-12-- Overall UAA Recommendations for D-Zn, D-Cd, and D-Pb

|  | D-Zn | D-Cd | D-Pb |
| --- | --- | --- | --- |
| UAA Recommendation | ELS Equation | Acclimated Adult Trout Equation | TVS |

The acute and chronic standards are approximately the same, with the chronic standard being slightly higher at increased hardness values.

Data collected from the Animas R,; Arkansas R.; Blue R., Eagle R.; French G.; Peru Crk.; & Snake R.

Due to the lack of studies conducted at high hardness levels that included brown trout, data from brook, and rainbow trout were also used in analysis.

Additional analysis was conducted on individual regression slopes, no statistical difference was noted.

Current TVS

Acute based on Equation : 0.938[e(0.7409(LnHardness)-4.719)]